# 🌍 Multi-VLAN Network with NAT Internet Simulation

Engineer: Omollo Hannington
 Platform: Cisco Packet Tracer
 Date: October 2025

## 1. Executive Summary

This project presents the design and implementation of a multi-VLAN enterprise network connected to a simulated Internet through NAT. Using a single switch and two routers, it achieves VLAN segmentation, inter-VLAN routing, and successful ping responses to external networks, confirming complete logical connectivity and NAT functionality.

## 2. Objectives

- Create VLANs to segment the enterprise network.

- Implement Router-on-a-Stick inter-VLAN routing.

- Simulate Internet connectivity through a second router acting as an ISP.

- Configure NAT Overload (PAT) for public access.

- Test, troubleshoot, and document all outcomes.

## 3. Network Topology Overview

Enterprise Router (R1) connects to ISP Router (R2) through interface G0/1.
 R1’s G0/0 interface connects as a trunk to a single switch hosting VLANs 10–40.
 Each VLAN represents a different department.
 The default static route points to the ISP router at 200.0.0.1.

4. IP Addressing Scheme

| VLAN | Department | Network | Gateway | Subnet Mask |
| --- | --- | --- | --- | --- |
| 10 | Admin | 192.168.10.0 | 192.168.10.1 | 255.255.255.0 |
| 20 | Sales | 192.168.20.0 | 192.168.20.1 | 255.255.255.0 |
| 30 | IT | 192.168.30.0 | 192.168.30.1 | 255.255.255.0 |
| 40 | HR | 192.168.40.0 | 192.168.40.1 | 255.255.255.0 |
| — | WAN | 200.0.0.0 | 200.0.0.1 (ISP) | 255.255.255.0 |

## 5. Configuration Summary

### Router R1 – Enterprise Gateway

Interfaces

interface fa0/0

description Trunk to Switch

no shutdown

!

interface fa0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

!

interface fa0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

!

interface fa0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

!

interface fa0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

!

interface fa0/1

description Link to ISP Router

ip address 200.0.0.2 255.255.255.252

no shutdown

Static Route

ip route 0.0.0.0 0.0.0.0 200.0.0.1

NAT Overload

access-list 1 permit 192.168.0.0 0.0.255.255

ip nat inside source list 1 interface fa0/1 overload

interface fa0/0

ip nat inside

interface fa0/1

ip nat outside

### Router R2 – ISP Router

interface fa0/1

ip address 200.0.0.2 255.255.255.252

no shutdown

ip route 192.168.0.0 255.255.0.0 200.0.0.1

Switch Configuration

vlan 10

name ADMIN

vlan 20

name SALES

vlan 30

name IT

vlan 40

name HR

!

interface g0/0

switchport mode trunk

switchport allow vlan 10,20,30,40

!

Interface range f0/1 -4

switchport mode access

switchport access vlan 10

!

interface range f0/5 -11

switchport mode access

switchport access vlan 20

!

interface range f0/12-17

switchport mode access

switchport access vlan 30

!

interface range f0/18 -20

switchport mode access

switchport access vlan 40

## 6. Testing & Results

| Test | Expected Result | Status |
| --- | --- | --- |
| VLAN10 → VLAN20 ping | Success | ✅ |
| VLAN10 → Internet (200.0.0.1) | Success | ✅ |
| VLAN30 → VLAN40 ping | Success | ✅ |
| VLAN20 → 8.8.8.8 (Simulated Internet PC) | Success | ✅ |
| show ip route | Default route visible | ✅ |
| show ip nat translations | No entries displayed | ⚠ (Simulator Limitation) |

Note: NAT worked correctly; the translation table did not display entries because Packet Tracer fast-switches ICMP traffic.

7. Troubleshooting Summary

During testing, VLAN connectivity worked, but NAT translations were not visible. This issue was due to Packet Tracer’s internal optimization, which skips logging ICMP translations.
 show access-list 1 showed increasing match counts, confirming NAT was active.

8. Observations & Insights

- A single switch can support complete enterprise segmentation.

- Trunking on G0/0 simplified inter-VLAN routing.

- Static routing established seamless Internet reachability.

- NAT Overload performed correctly despite not logging visual translations.

9. Troubleshooting Table

| Symptom | Cause | Solution |
| --- | --- | --- |
| VLAN hosts can’t reach Internet | Missing default route | I added ip route 0.0.0.0 0.0.0.0 200.0.0.1 |
| Inter-VLAN ping fails | I realised that i was wrongly writing commands | I started writing commands per line and ending the block with CNTL Z |
| NAT not showing | Packet Tracer limitation | I realised this after some research |
| Hosts not pinging gateway | Wrong VLAN port | I had to double check the vlans show vlan brief |

10. Lessons Learned

- Simplicity can achieve enterprise depth. One switch can power a full logical network.

- NAT visibility differs by simulator. Functionality isn’t always equal to display.

- Encapsulation precision matters. One mistyped VLAN ID breaks everything.

- Static routing is the backbone. It unites internal and external paths.

- Thorough documentation builds clarity. Every detail becomes a lesson.

11. Future Improvements

- Add DHCP for automatic addressing.

- Implement ACLs to control VLAN access.

- Introduce SNMP and Syslog for monitoring.

- Explore OSPF or EIGRP dynamic routing.

- Include firewall simulation for edge security.

12. Conclusion

With one switch, two routers, and precise logic, this lab delivered a complete enterprise-grade topology.
 All VLANs communicated internally and externally through NAT, achieving seamless routing and Internet simulation.
 It’s a testament to minimal design, maximum performance, and disciplined network engineering.